# Basic GPIO Notes with Registers

## How to Enable Clock for GPIO Ports

To use any GPIO port, you must first enable its clock using the RCC register:

RCC->AHB1ENR — Enable the clock for the required GPIO port by setting its corresponding bit.

## 1. Basic Purpose

• GPIO pins are used to interface the microcontroller with external devices (e.g., Motors, Buttons, LEDs, Sensors).

• Each pin can be configured as Input, Output, Analog, or Alternate Function.

## 2. GPIO Modes

• Input mode (floating, pull-up, pull-down)

• Output mode (push-pull/open-drain)

• Alternate Function mode

• Analog mode

## 3. Output Types

• Push-pull → Drives HIGH and LOW actively

• Open-drain → Can only drive LOW; requires pull-up resistor

## 4. GPIO Registers

• MODER → Selects pin mode

• OTYPER → Push-pull / open-drain

• OSPEEDR → Output speed

• PUPDR → Pull-up / pull-down

• IDR → Input data register

• ODR → Output data register

• BSRR → Atomic set/reset register

• LCKR → Lock pin configuration

• AFRL / AFRH → Alternate function selection

## 5. Input Pin Behavior

• Floating input is unpredictable

• Pull-up keeps input HIGH when unconnected

• Pull-down keeps input LOW when unconnected

# Register Descriptions (Simple Explanation)

## 1) MODER – Mode Register

• 32‑bit register.

• Each pin uses 2 bits to select mode.

• Can configure up to 16 GPIO pins.

Mode bits:

| Bits | Description |
| --- | --- |
| 00 | Input mode (reset state) |
| 01 | Output mode |
| 10 | Alternate function |
| 11 | Analog mode |

Example: Configure PA0 as Output:

GPIOA->MODER &= ~(3 << (0 * 2));

GPIOA->MODER |=  (1 << (0 * 2));

## 2) OTYPER – Output Type Register

• 32‑bit register (0–15 used, 16–31 reserved).

• Selects output type.

## 3) OSPEEDR – Output Speed Register

• 32‑bit register.

• Each pin uses 2 bits to select speed.

## 4) PUPDR – Pull-up/Pull-down Register

• 32‑bit register.

• Each pin uses 2 bits to configure pull resistors.

## 5) IDR – Input Data Register

• 32‑bit register, bits 0–15 used (read‑only).

• Reads the state of input pins.

## 6) ODR – Output Data Register

• 32‑bit register, bits 0–15 used (read/write).

• Used to set/reset output pins.

## 7) BSRR – Bit Set/Reset Register

• Atomic pin control register.

• Bits 0–15 set pins; bits 16–31 reset pins.

## 8) AFR – Alternate Function Register

• 32‑bit register.

• Each pin uses 4 bits for selecting alternate function.

• AFRL → Pins 0–7, AFRH → Pins 8–15.

## 9) LCKR – Lock Register

• 32‑bit register.

• Used to lock GPIO pin configuration until reset.

• Bits 0–15 select pins to lock, bit 16 (LCKK) is lock key.